# Altitude Measurement Low-Level Design (LLD)

Version: 1.0

Date: 17-01-2026

Prepared by: Arunya V. A

## 1.Purpose

The Purpose of this document is to provide a low level design of the Altitude measurement. It describes modules into function, its prototypes  and ensures the system meets the requirements described in the Altitude Measurement HLD. This document aims to reduce the potential errors that come in the coding phase and for future enhancements.

## 2. Functional Decomposition

## 2.1 Sensor Management

Sensor management initializes the sensor module, reads data from and pushes data in the message queue. Following are the details of available functions and subroutines.

#### 2.1.1 SensorInitialization()

Initialization of sensor module.

#### 2.1.2 SensorAltitudeMeasure()

Act as key entry point, initialize IPC and concurrent execution unit.

#### 2.1.3 SensorReadHandler()

Acting as the entry point for sensor Handler, this function manages the data flow of sensor module to message queue.

#### 2.1.4 SensorDataRead()

This subroutine is responsible for reading data from the sensor module. Using [C] protocol, it fetches data from the  module.

### 2.2 Validation Engine

The Validation engine performs the processing of data and validation. Process the message queue and validate with [THRESHOLD_MIN, THRESHOLD_MAX].Following are the details of available functions and subroutines.

#### 2.2.1 ProcessSensorData()

Serve as a validate engine,  it de-queues the message packet,  extract the raw data and perform validation and set event signal based on the processed output.

#### 2.2.2 ProcessValidateAlititude()

This will check the altitude value we pass with  [THRESHOLD_MIN, THRESHOLD_MAX].Return value as per the conditions.

### 2.3 Alarm Management

Monitors the event system group and manages alarm triggering.

#### 2.3.1 AlarmInfoProcessHandler()

Validate the event flag and trigger the alarm.

## 3. Detailed Function Descriptions

### 3.1.Function Prototypes

This module describes the prototypes of above-mentioned functions.

- bool SensorInitialization(uint32_t ulFrequency)

- void SensorAltitudeMeasure(void);

- void* SensorReadHandler(void* arg);

- static uint32_t SensorDataRead(void);

- void* ProcessSensorData(void* arg);

- static bool ProcessValidateAlititude(uint32_t ulAltitudeData);

- void* AlarmInfoProcessHandler(void* arg);

### 3.2 Function pseudocode

#### 3.2.1 SensorInitialization()

This will be used to set frequency of sensor module.

FUNCTION  SensorInitialization()

DECALRE A BOOLEAN  VARIABLE   blReturnSuccess , SET AS FASLE;

CHECK ulFrequency   NOT EQUAL TO  ZERO

ASSIGN ulFrequency[INPUT PARAMETER]  TO  MODULE  FREQUENCY

SET BOOLEAN blReturnSuccess AS  true;

RETURN BOOLEAN  blReturnSuccess;

#### 3.2.1 SensorAltitudeMeasure ()

Key entry point which called from main()

FUNCTION SensorAltitudeMeasure()

DECLARE MESSAGE_HANLDERS, EVENT_HANDLER, thread1,thread2,thread3;

SET MSG_QUEUE_PARAMS;

SET THREAD_PARAMS;

/*Message queue implementation*/

MESSAGE_HANLDER = CREATE_MSG_QUEUE(MSG_QUEUE_PARAMS))

IF MESSAGE_HANLDER IS NOT VALID

PRINT ERROR MESSAGE

RETURN

/*Event handler implementation*/

EVENT_HANDLER = CREATE_EVENT())

IF EVENT_HANDLER IS NOT VALID

PRINT ERROR MESSAGE

RETURN

CREATE_THREAD(thread1,SensorReadHandler);

CREATE_THREAD(thread2,ProcessSensorData);

CREATE_THREAD(thread3,AlarmInfoProcessHandler);

IF thread1 AND thread2 AND thread3 ARE VALID

WAIT FOR THREAD TO EXECUTE Its FUNCTION

ELSE

PRINT ERROR MESSAGE

RETURN

#### 3.2.3 SensorReadHandler()

Manages the data flow of sensor module and message queue

FUNCTION SensorReadHandler()

DECLARE STATIC UNSIGNED INTEGER VALUE ulAltitudeValue , SET AS ZERO;

DECLARE Timervariable AND ASSIGN TIME_INTERVAL;

DO IN A WHILE LOOP

WAIT  Timervariable  REACHES TO ZERO

USE SensorDataRead() FUNCTION TO READ DATA AND ASSIGN TO         ulAltitudeValue

ASSIGN  ulAltitudeValue TO  MESSAGE_PACKET;

PUSH MESSAGE_PACKET  TO  QUEUE;

RESET THE Timervariable BY  TIME_INTERVAL;

#### 3.2.4 SensorDataRead();

Read data from sensor module

FUNCTION SensorDataRead(void)

DECLARE UNSIGNED INTEGER  ulAltitudeValue, SET AS ZERO;

READ DATA FROM SENSOR

ASSIGN DATA  TO  ulAltitudeValue

RETURN THE VARIABLE ulAltitudeValue

#### 3.2.5 ProcessSensorData

Deque the data , validate and set event flag

FUNCTION ProcessSensorData(void* arg)

DECLARE UNSIGNED INTEGER  ulAltitudeValue, SET AS ZERO;

WHILE MONITOR THE SYSTEM

DEQUEUE DATA

ASSIGN DATA TO ulAltitudeValue

IF (  FUNCTION ProcessValidateAlititude(ulAltitudeValue) RETURN TRUE)

SET EVENT BIT EVENT_ALARM_BIT;

#### 3.2.6 ProcessValidateAlititude()

Validate the altitude with given ranges and threshold values.

FUNCTION  ProcessValidateAlititude(uint32_t ulAltitudeValue)

DECALRE A BOOLEAN  VARIABLE   blSetAlarm, SET AS FASLE;

IF MIN RANGE IS GREATER THAN ulAltitudeValue AND MAX_RANGE LESS  THAN ulAltitudeValue THEN DO

IF THRESHOLD_MIN LESS THAN  ulAltitudeValue OR THRESHOLD_MAX GREATER THAN ulAltitudeValue THEN DO

SET  blSetAlarm AS  true

RETURN  blSetAlarm

#### 3.2.7 AlarmInfoProcessHandler()

Perform alarm triggering by checking event signal.

FUNCTION AlarmInfoProcessHandler(void* arg)

WHILE MONITOR THE SYSTEM

IF EVENT ALARM BIT SET THEN DO

TRIGGER ALARM UPTO ALARM_TIMEOUT

## 4. Data Structures

### 4.1 Data types

| Variables | Purpose |
| --- | --- |
| unint32_t ulFrequency | Used for setting sensor module frequency |
| static unint32_t ulAltitudeValue | To read Altitude value from sensor module |
| static bool blReturnSuccess | Used for returning status from various functions. |

#### 4.2 Constants

| TIME_INTERVAL | 1 sec |
| --- | --- |
| EVENT_ALARM_BIT | 1UL |
| MIN RANGE | 0 |
| MAX_RANGE | 5000 |
| THRESHOLD_MIN | 100 |
| THRESHOLD_MAX | 2000 |
| ALARM_TIMEOUT | 10 msec |

## 5. Error Handling

#### 5.1 Error handling

- Validate the thread creation process.

- Validate message queue creation.

- Validate event creation.

- Check for the overflow.

- Check CPU utilization.

#### 5.2 Boundary Checks

- Validate altitude values from sensor by using MIN_RANGE AND MAX_RANGE.

- Proper Queue Size creation.

#### 5.3 Exception mechanism

- Defined MIN_RANGE AND MAX_RANGE for restrict garbage values.

- Checked NULL  and returned values to avoid system crashes.

## 6. Timing and Performance

#### 6.1 Timing requirements

- Check the sensor execution time with our defined Interval.

- Monitor the pushing and popping of message from queue.

- Overall system performance.

#### 6.2 Execution Constraints

- To check and prevent race conditions.

- Message overflow

- Sensor failure

#### 6.3 Memory usage

- Stack allocation

- CPU utilization

## 7. Test Points

- Check by changing the Threshold Values and Ranges.

- Changed the interval and monitor the system performance and accuracy.

- Sensor failure condition.

## 8. Traceability

| References(Altitude measurement HLD.docx) | Reference ID |
| --- | --- |
| Module Description | 4 |
| Interface design | 5 |
| Data Flow | 6 |
| Error handling and fault management | 7 |